#include <DHT.h>

// Pin Configuration

#define DHTPIN 4          // DHT Sensor connected to Pin D4

#define DHTTYPE DHT11     // Standard DHT11 configuration

#define PIRPIN 14         // PIR Motion Sensor connected to Pin D14

#define RELAYPIN 12       // Optocoupler Relay control connected to Pin D12

#define CURRENTPIN 34     // Analog Current Sensor channel linked to Pin D34

// Constants for Power Calculation

const float LINE_VOLTAGE = 220.0; // Standard Household AC Voltage in India

DHT dht(DHTPIN, DHTTYPE);

void setup() {

Serial.begin(115200);

// Initialize Peripherals & Safe Pin Directions

dht.begin();

pinMode(PIRPIN, INPUT);

pinMode(RELAYPIN, OUTPUT);

// Keep load safely OFF initially

digitalWrite(RELAYPIN, LOW);

Serial.println("Smart Energy System Initialized...");

}

void loop() {

// Phase 1: Sensor Data Acquisition

int occupancy = digitalRead(PIRPIN);

float temp = dht.readTemperature();

// Phase 2: Automation & Relay Control Mechanism

// If human occupancy is detected, turn ON the appliance channel via relay

if (occupancy == HIGH) {

digitalWrite(RELAYPIN, HIGH);

} else {

digitalWrite(RELAYPIN, LOW);  // Turn off load to eliminate idle energy wastage

}

// Phase 3: Current Sampling and Mathematical Power Computations

int analogRaw = analogRead(CURRENTPIN);

// Engineering Calibration factor to scale raw bits to actual current amperes

float currentAmps = (analogRaw * (3.3 / 4095.0)) * 5.0;

// Core Power Formula Calculation: P = V * I

float powerWatts = LINE_VOLTAGE * currentAmps;

// Output Telemetry Stream to Local Serial Monitor / IoT Hub Logs

Serial.print("Occupancy Status: "); Serial.println(occupancy ? "Room Occupied" : "Room Vacant");

Serial.print("Ambient Temp: "); Serial.print(temp); Serial.println(" C");

Serial.print("Calculated Power Demand: "); Serial.print(powerWatts); Serial.println(" Watts");

Serial.println("------------------------------------");

delay(2000); // 2-second sampling interval

}